| 26. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Which best describes the definition of Type I “Small Appliance” as defined by the EPA? a.; Systems manufactured, charged and hermetically sealed with five pounds or less of refrigerant; c.; any appliance with more than 5 pounds of refrigerant. b.; refrigerators, freezers, room air conditioners and central air-conditioners; d.; any appliance charged with less than two pounds of refrigerant. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For service and repair and maintenance of small appliances, recovery equipment manufactured after _____ must be certified by an EPA approved organization. a.; July 1, 1992; c.; January 1, 1995. b.; August 12, 1992; d.; November 15, 1993. |
| 28. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | For small appliance use, the recovery unit manufactured after November 15, 1993, must be capable of recovering: a.; 80% of the refrigerant when the compressor is not operating; c.; 80% of the refrigerant when the compressor is operating or achieve a 4” vacuum under ARI-1993. b.; 90% of the refrigerant when the compressor is operating or achieve a 4” vacuum under ARI 740-1993; d.; A & B above. |
| 29. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The sale of Class I and Class II refrigerants is restricted to technicians certified by an EPA approved program after: a.; July 1,1992; c.; Aug. 12, 1993. b.; November 15, 1993; d.; November 14, 1994. |
| 30. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The release of vapor from the top of a charging cylinder when filling should: a.; be vented to the atmosphere; c.; not be vented and should be recovered. b.; be vented if the quantity does not exceed 3 pounds; d.; be vented but not be inhaled. |
| 31. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Should the regulations of the Clean Air Act (CAA) change after a technician becomes certified: a.; the technician must take a new exam to remain certified; c.; it will be the technician’s responsibility to comply with any changes in the law. b.; all technicians that previously passed with a grade over 80% will be “grandfathered”; d.; the technician will be retested with a required passing grade of 90%. |
| 32. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Containers of HFC-134a are colored: a.; green; c.; light blue. b.; white; d.; Yellow. |
| 33. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | System dependant refrigerant recovery of small appliances: a.; does not require an operating compressor; c.; recovers refrigerant into a non-pressurized container. b.; requires 80% of the charge to be recovered; d.; all of the above. |
| 34. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Mixed refrigerant sent to reclaim centers: a.; may be disposed of at owners expense; c.; can be reused as is. b.; can be reclaimed and resold; d.; just needs to be filtered. |
| 35. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Before disposing of a small appliance containing CFC-12, it is necessary to: a.; pressurize with nitrogen; c.; turn upside down. b.; recover the refrigerant; d.; thoroughly leak check. |
| 36. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Hydrochloric and hydrofluoric acids are: a.; caused from CFCs or HCFCs in the presence of moisture being exposed to high heat; c.; to be avoided when handling with bare hands. b.; damaging to the windings found in hermetic compressors; d.; all of the above. |
| 37. |  |  |  |  |  |  |  |  |  |  |  |  |  | A system has been operating with a complete restriction at the capillary tube inlet, what access is required for recovery? a.; on access valve on the low side of the system; c.; one access valve on the high side. b.; two valves, high and low side; d.; one access on the evaporator and one on the low side. |
| 38. |  |  |  |  |  |  |  |  |  |  |  |  | Which of the following refrigerants can be mixed? a.; R-12 & R-134a; c.; R-22 & R-502. b.; R-12 & R-11; d.; None of the above. |
| 39. |  |  |  |  |  |  |  |  |  |  |  | CFC and HCFC refrigerants cause suffocation because: a.; they contain acidic substances; c.; they are lighter than air and will rise. b.; they are heavier than air and displace oxygen; d.; the smell terrible preventing you from breathing. |
| 40. |  |  |  |  |  |  |  |  |  |  | A nitrogen tank should always be equipped with a(n): a.; pressure regulator; c.; low loss fitting. b.; EPR valve; d.; air purger. |
| 41. |  |  |  |  |  |  |  |  |  | Nitrogen used for brazing purposes, holding charges, or leak checking: a.; must be recovered; c.; may be vented to the atmosphere. b.; should not be used. It can saturate the system with moisture; d.; could be recovered passively. |
| 42. |  |  |  |  |  |  |  |  | Line tapping and piercing valves should always be removed from a system after the repairs are completed because: a.; they restrict the flow of refrigerant; c.; the neoprene gasket will eventually fail and cause a leak. b.; they are costly; d.; they destroy the appearance. |
| 43. |  |  |  |  |  |  |  | CFCs will not be manufactured in the USA after: a.; 2000; c.; 2005. b.; 1995; d.; 1996. |
| 44. |  |  |  |  |  |  | The law states that leaks in small appliances: a.; must be repaired; c.; do not have to be repaired but it is a good practice to repair whenever possible.. b.; be repaired if the system contains more than five (5) pounds of refrigerant; d.; be repaired if the leak rate exceeds two (2) pounds a year. |
| 45. |  |  |  |  |  | To work on small appliances after November 14, 1993, a technician must be certified as: a.; Type I; c.; Universal. b.; Type II; d.; A or C. |
| 46. |  |  |  |  | The required service aperture usually found on small appliances is: a.; the three-way service valve; c.; the schrader valve. b.; the process stub; d.; the door handle. |
| 47. |  |  |  | Package terminal heat pumps with less than five (5) pounds of refrigerant must be serviced by technicians with a _____ certification. a.; Type I; c.; Type III. b.; Type II; d.; Type II or III. |
| 48. |  |  | Which refrigerants have a “0” ODP? a.; CFCs; c.; HFCs. b.; HCFCs; d.; All of the above. |
| 49. |  | Disposable refrigerant cylinders should be disposed of by: a.; rendering useless and sent to salvage; c.; using for a recovery cylinder. b.; taking to a service station and using for an air tank; d.; refill with other refrigerants. |
| 50. | Which refrigerant is most commonly used today as a replacement for R-12 in newly manufactured small appliances? a.; r-22; c.; r-134a. b.; r-502; d.; r-123. |