TYPE 1

1) For the maintenance, service or repair of small appliances, recovery equipment

manufactured after_________ must be certified by an EPA approved organization?

- July 1, 1992

- August 12, 1992

- January 1,1995

- November 15, 1993

2) Which best describes the definition of Type 1 “Small appliance,” as defined by

EPA?

- System manufactured, charged and hermetically sealed with 5 pounds or less of refrigerator

- Refrigerators, freezers, room air conditioners and central air conditioners

- Any appliance charged with more than 5 pounds of refrigerant

- Any appliance charged with less than 2 pounds of refrigerant

3) For small appliance use, the recovery equipment manufactured after November

15, 1993, must be capable of recovering?

a) 80% of the refrigerant when the compressor is not operating or 4” vacuum

under ARI 740, 1993

b) 90% of the refrigerant when the compressor is operating or achieve 4” vacuum

under ARI 740, 1993

c) 99% of the refrigerant regardless of compressor operation and achieve a 10”

vacuum under 740, 1993

- Both a and b

4) The sale of Class 1 and Class 2 refrigerant will be restricted to technicians

certified by an EPA-approved program after?

- July 1, 1992

- November 15,1993

- August 12, 1993

- November 14,1994

5) The release of vapor from the top of a charging cylinder, when filling, should?

a) Be vented

b) Be vented if the quantity does not exceed 3 pounds

c) Not be vented and should be recovered

d) Be vented, but not inhaled

6) Should regulations of the Clean Air Act (CAA) change after a technician is

certified?

- The technician must take a new test to be recertified

- All technicians who previously passed with an 80% will be grandfathered

- It will be the technicians responsibility to learn and comply with future changes in the law

- The technicians must be retested and pass the exam with a 84%

7) Containers of refrigerant R-134a are color coded?

a) Green

b) White

c) Yellow

d) Light-blue

8) System dependent (passive) refrigerant recovery of small appliances?

a) Do not require an operating compressor

b) Requires 80% of the refrigerant to be recovered

c) Recovers refrigerant on a non-pressurized container

d) All of the above

9) Mixed refrigerants that are sent to a reclaim center____________?

a) Will be disposed of at the owners expense

b) Can be reclaimed and resold

c) Can be reused in its currant state

d) Just needs filtering

10) Before disposing of a small appliance containing R-12, it is necessary to ?

a) Pressurize with nitrogen

b) Recover the refrigerant

c)  Turn up side down

d) Thoroughly leak checked

11) Hydrochloric and hydrofluoric acids are?

a) Caused from CFC’S or HCFC’S in the presence of moisture being exposed to

high heat

b) Damaging to the windings found in the hermetic refrigeration and air

conditioning systems

c) To be avoided when handling with bare hands

d) All of the above

12) A system has been operating with a complete restriction at the capillary tube

inlet, what access is required for recovery?

- 1 access valve on the low side of the system

- 2 access valves, high and low side of the system

- 1 access valve on the high side of the system

- 1 access valve on the evaporator, and one on the low side

13) Which of the following refrigerants can be mixed?

a) R-12 and R-134a

b) R-12 and R-11

c) R-22 and R-502

d) none of the above

14) CFC and HCFC refrigerants can cause suffocation because?

a) They contain acidy substance
      b) They are heavier than air and displace oxygen

c) They are lighter than air and will raise

d) They smell terrible preventing you from breathing

15) A nitrogen tank should always be equipment with a ?

a) Pressure regulator

b) Evaporator pressure regulator

c) a low loss fitting

d) An air purger

16) Nitrogen used for brazing processes, holding charges, or leak checking?

a) Must be recovered

b) Should not be used, it will saturate the system with moisture

c) May be vented into the atmosphere

d) Can be recovered passively

17) Line tapping and piercing valves should always be removed from a system after

the repairs are completed because?

a) They restrict the flow of refrigerant

b) They are costly

c) The Neoprene gaskets will eventually fail causing a leak

d) They destroy the appearance

18) CFC’S will not be manufactured in the United States after?

a) 2000

b) 1995

c) 2005

d) 1996

19) The law states that leeks in small appliance?

a) Must be repaired

b) Be repaired if the system contains more than 5 pounds of refrigerant

c) Do not have to be repaired, but should be repaired

d) Be repaired if the leak rate exceeds 2 pounds a year

20) To Work on small appliances after November 14,1994 a technician must be

certified as?

- Type 1

- Type 2

- Universal

- Both a and c

21) The required aperture typically found on small appliance systems are?

a) The three-way service valve

b) The process stub

c) The Schrader valve

d) The door handle

22) Packaged terminal heat pumps with less than 5 pounds of refrigerant must be

serviced by certified?

- Technicians with Type 1

- Technicians with type 2

- Technicians with type 3

- Technicians with type 2 or 3

23) Which refrigerants have “0” ODP?

a) CFC’S

b) HCFC’S

c) HFC’S

d) All of the above

24) Disposable refrigerant cylinders should be disposed of by?

a) Rendering useless and sent to salvage

b) Taking to the service station and used for an air tank

c) Recovery cylinder

d) Refill with other refrigerant

25) Which refrigerant is a strong candidate for replacing CFC (R-12) in small

appliances?

- R-22

- R-502

- R-123

- R-134a

26) Type 1 certification defines small appliances as products that are fully

manufactured, charged, and hermetically sealed in a factory with ___ pounds or

less of refrigerant?

- 10

- 5

- 7

- 15

27) A drop in replacement for R-12 is?

a) R-22

b) R-123

c) R-134a

d) None of the above

28) For small appliance use, a recovery device manufactured after November 15,

1993 must be capable of?

a) Recovering 80% of the refrigerant when the compressor is not operating or

reaching 4” vacuum under ARI 740

b) Recovering 90% of the refrigerant when the compressor is not operating or

reaching 4” vacuum under ARI 740

c) Nothing specific, as there is no standard for recovery in small appliances

d) All of the above

29) For small appliance use, a recovery device manufactured before November 15,

1993 must be capable of?

a) Recovering 80% of the refrigerant when the compressor is operating

b) Recovering 80% of the refrigerant when the compressor is not operating

c) Being operated by a certified technician after November 14, 1994

d)  All of the above

30) A type 1 technician can service a package unit air conditioner with a charge not

exceeding?

- 5 lb.

- 10 lb

- 15lb

- There is no limit as long as the unit is a package unit

31) In order to service small appliances, a technician must be certified as?

a) Type1

b) Type 2

c) Type 1 or universal

d) All of the above

32) Refrigerants can be mixed in the recovery cylinder?

a) When you do not know what type of refrigerant you are recovering

b) All the time if the cylinder s marked

c) Never

d) Only when the refrigerant is going to be reclaimed

33) When filling a dial-a-charge, the refrigerant from the top may?

a) Be vented

b) Not be vented and must be recovered

c) Cause an accidental release and is not important

d) Be vented up to a quantity of 1lb.

34) Once you become certified, the law states you must repair leeks in all small

appliances?

- True

- False

- Depends on the size of the leek

- Only if the refrigerant is a CFC

35) Active recovery uses the appliance’s compressor to recover the refrigerant?

a) True

b) False

36) Passive recovery is system dependent recovery?

a) True

b) False

37) If the compressor is not running in passive recovery, you are required to?

a) Recover from both low and high side

b) Recover from high side only

c) Recover from low side only

d) Not recover, because it will be alright to vent

38) A container of R-12 is color coded?

a) Light-blue

b) Orange

c) White

d) Green

39) A container of R-134a is color coded?

a) Light-blue

b) Orange

c) White

d) Green

40) Passive recovery?

a) Capture refrigerant in non-pressurized container

b) Can be assisted by adding heat to the refrigerant

c) Does not require a working compressor

d) All of the above

41) Which group of refrigerants does not harm the ozone layer?

a) CFC’s

b) HCFC’S

c) HFC’S

d) All of the above

42) When you are finished with a disposable cylinder?

a) dispose of it as trash

b) refill the cylinder as long as the refrigerant is not a CFC

c) recover any refrigerant, render the cylinder unusable and recycle it

d) use it as an air tank

43) Before disposing of an appliance containing a CFC you must?

a) seal any leaks so that any remaining refrigerant will not escape

b) recover the refrigerant

c) add nitrogen to dilute any remaining refrigerant

d) obtain a CFC disposal permit

44) CFC’s will no longer be manufactured after what year?

a) 1995

b) 1998

c) 2000

d) 2005

45) All recovery machines must?

a) meet standards set by EPA

b) have UL approval

c) meet ARI standard 700

d) be approved by ASHRAE

47) R-12 and R-22 can cause suffocation because?

a) they are heavier than air and displace oxygen

b) they contain chemicals that attack the lungs

c) they are lighter than air

d) they cannot cause suffocation unless you inhale directly from the hose

48) When brazing or exposing refrigerants to open flames?

a) R-12 and R-22 can form phosgene gas

b) R-12 and R-22 can form hydrochloric and hydrofluoric acid

c) insure you are working in a well ventilated area

d) all of the above

49) It is legal to vent refrigerant until November 14, 1994 as long as you are certified

by that date?

a) true

b) false