Model hints:

Place a large piece of paper with your set drawn out to scale on the board before you begin cutting.  Be sure all your furniture fits in the set wall boundaries

Use a T-square or framing square for straight walls.

Foam core comes in a variety of colors and some textured looks.

Score the foam board , then break it in the direction of the cut for exterior corners.  Score the opposite side for interior corners.

Be careful when painting foam core.  It likes to warp when it gets too wet.  You can always put colored paper or wallpaper on a wall.

Don’t forget chair rails to “break up” tall walls.

Draw windows on the walls in pencil to check their height.

Cut openings for doors & windows carefully.  Err on the side of too small an opening.  You can always remove excess.  It’s hard to add if the opening is too large.

Don’t forget maskings!

 If you use spray paint on foam core, the styrofoam will still melt. 

