Quarter 3 Homework 1

- Determine the time constant and total time transition for the values below:

- R = 750 Ω, L = 100 μH

- For part a, determine the time required for VR to reach +4V on the rise curve with Vs = 0-10V.

- R = 100 kΩ, L = 2 mH

- For part c, determine the time required for VR to reach +18V on the rise curve with Vs = 0-25V.

- R = 3kΩ, C = 0.22 μF

- For part e, determine the time required for Vc to reach +6V on the rise curve with Vs = 15V.

- R =  5.1kΩ, C = 100 nF

- For part g, determine the time required for Vc to reach +15V on the rise curve with Vs = 18V.

- An RL switching circuit has values of R = 33 Ω, L = 100 μH, Ipk = 75 mA. Determine the value of the circuit current (It) at the following times:

- 1.4 μs after the start of the current rise.

- 2 μs after the start of the current decay.

- An RC switching circuit has values of R = 750 Ω, C = 220 μF, and Vpk = 18 V. Determine the value of Vc at the following times:

- 100 ms after the start of the Vc rise cycle.

- 300 ms after the start of the Vc decay cycle.

- An RC switching circuit has values of R = 120 Ω, C = 47 μF, and Vpk = 8 V. Determine the value of Vc at the following times:

- 8 ms after the start of the Vc rise cycle.

- 4 ms after the start of the Vc decay cycle.