| 1. | Which refrigerants contain the most chlorine? a.; HCFCs; c.; CFCs. b.; HFCs; d.; Ammonia. |
| --- | --- |
| 2. | As of what date did it become unlawful to intentionally release Class I and Class II refrigerants into the atmosphere? a.; July 1, 1992; c.; November 14, 1994. b.; July 1, 1986; d.; January 1, 1996. |
| 3. | Which refrigerants have the lowest ozone depletion potential? (ODP)? a.; CFCs; c.; HFCs. b.; HCFs; d.; Ternary blends of HCFCs. |
| 4. |  | The molecule found in CFC and HCFC refrigerants that destroys ozone in the atmosphere is: a.; fluorine; c.; hydrogen. b.; carbon; d.; chlorine. |
| 5. |  | Stratospheric ozone consists of a.; one oxygen atom; c.; three oxygen atoms. b.; two oxygen atoms; d.; four oxygen atoms. |
| 6. |  | stratospheric ozone depletion is a ______ problem. a.; American; c.; local. b.; global; d.; United Nations. |
| 7. |  | the gas in the stratosphere that protects the earth from ultraviolet radiation is: a.; radon; c.; ozone. b.; sulfur dioxide; d.; carbon dioxide. |
| 8. |  | Which refrigerant is a CFC? a.; R-134a; c.; R-22. b.; R-123; d.; R-12. |
| 9. |  | Which refrigerant is a HCFC? a.; R-134a; c.; R-22. b.; R-123; d.; R-12. |
| 10. |  | Which refrigerant is an HFC? a.; R-134a; c.; R-22. b.; R-123; d.; R-12. |
| 11. |  | Which refrigerant contains no chlorine? a.; R-134a; c.; R-22. b.; R-123; d.; R-12. |
| 12. |  | The molecule found in the stratosphere that indicates that ozone depletion is taking place is: a.; carbon monoxide; c.; chlorine monoxide. b.; carbon dioxide; d.; radon. |
| 13. |  | Which of the following would violate the Clean Air Act (CAA)? a.; knowingly releasing Class I or Class II substances; c.; failing to reach required evacuation levels before opening a system a system for service. b.; failing to keep or falsifying records; d.; all of the above. |
| 14. |  | One time (disposable) refrigerant cylinders are used for: a.; sending refrigerant for reclaim; c.; recycling refrigerant. b.; containing recovered refrigerant; d.; virgin refrigerant. |
| 15. |  | The maximum safe fill level of a refrigerant cylinder is ____% liquid. a.; 60; c.; 80. b.; 70; d.; 90. |
| 16. |  | The time it takes to properly evacuate and dehydrate a refrigeration system is determined by: a.; the total volume of the system; c.; the ambient temperature. b.; the amount of non-condensables in the system; d.; all of the above. |
| 17. |  | A system that fails to hold a vacuum after the evacuation process: a.; has a restriction; c.; may have a leak. b.; may contain moisture; d.; B & C. |
| 18. |  | To RECOVER refrigerant is to: a.; remove the refrigerant from a system in any condition and store it in an external container without necessarily processing or testing; c.; reprocess to new refrigerant standards. b.; reduce contaminants through oil separation and filtering; d.; remove refrigerant and change ownership. |
| 19. |  | The process that reduces contaminants in used refrigerant by oil separation and single or multiple passes through devices such as replaceable filter core driers which reduce moisture, acidity and particulate matter is called: a.; restore; c.; recycle. b.; recovery; d.; reclaim. |
| 20. |  | The synthetic lubricant used with HCFC based ternary blends is a.; alkyl-benzene; c.; mineral. b.; PAG; d.; ester. |
| 21. |  | HFC-134a is a drop-in replacement for: a.; R-12; c.; R-11. b.; R-22; d.; HFC-134a is not a drop-in refrigerant. |
| 22. |  | The state and condition of the refrigerant entering the condenser is a : a.; sub-cooled liquid; c.; superheated vapor. b.; sub-cooled vapor; d.; superheated liquid. |
| 23. |  | The state and condition of the refrigerant leaving the condenser is a : a.; sub-cooled liquid; c.; superheated vapor. b.; sub-cooled vapor; d.; superheated liquid. |
| 24. |  | The component of a refrigeration system that changes a low pressure vapor into a high pressure vapor is the: a.; condenser; c.; evaporator. b.; metering device; d.; compressor. |
| 25. |  | Refrigerant cylinders should be shipped: a.; upside down; c.; parallel with the truck axle. b.; in the vertical, upright position; d.; in the original carton. |