| 51. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Which refrigerant can be used for leak detection as a trace gas and pressurized with nitrogen? a.; R-12; c.; R-22. b.; R-11; d.; R-115. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 52. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Traces of oil around the sight glass inlet fitting of a refrigeration system might be an indication of: a.; a leak; c.; an overcharge. b.; excessive oil in the system; d.; a restriction at the TXV. |
| 53. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | A condition of excessive superheat at the evaporator outlet and compressor inlet of an air conditioning system could be an indication of: a.; high head pressure; c.; insufficient water. b.; a dirty condenser; d.; low refrigerant charge. |
| 54. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Type II classification, as identified by the EPA, applies to what equipment? a.; small appliances with five (5) pounds or less refrigerant; c.; low pressure chillers. b.; refrigerators, freezers, vending machines; d.; split air conditioning systems with more than five (5) pounds of refrigerant charge. |
| 55. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Front seating the suction service valve will close the a.; Suction inlet to the valve; c.; gauge port and suction inlet. b.; gauge port; d.; valve port to compressor. |
| 56. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The required level of evacuation for recovery equipment manufactured before November 15, 1993 0n a system with an R-22 charge of less than 200 pounds is: a.; 0” Hg; c.; 10” Hg. b.; 4” Hg; d.; 25 “ Hg. |
| 57. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The required level of evacuation for recovery equipment manufactured after November 15, 1993 0n a system with an R-22 charge of less than 200 pounds is: a.; 0” Hg; c.; 10” Hg. b.; 4” Hg; d.; 25 “ Hg. |
| 58. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | The required level of evacuation for recovery equipment manufactured after November 15, 1993 0n a system with an R-12 charge of less than 200 pounds is: a.; 0” Hg; c.; 10” Hg. b.; 4” Hg; d.; 25 “ Hg. |
| 59. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Industrial process and commercial refrigeration equipment with an annual leak rate of ____% requires repair under EPA regulations. a.; 0; c.; 35. b.; 15; d.; 50. |
| 60. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Comfort cooling chillers and all other equipment with an annual leak rate of ____% requires repair under EPA regulations. a.; 0; c.; 35. b.; 15; d.; 50. |
| 61. |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Recovery time can be shortened by: a.; recovering as much liquid as possible in the initial stages; c.; cooling the recovery tank in ice water. b.; using short hoses; d.; all of the above. |
| 62. |  |  |  |  |  |  |  |  |  |  |  |  |  | The majority of the liquid to be recovered from a system will be found in the: condenser a.; receiver (when applied); c.; low side. b.; condenser; d.; Evaporator. |
| 63. |  |  |  |  |  |  |  |  |  |  |  | It becomes the owners responsibility to maintain records of all refrigerant added to units that contain more than ____ pounds of refrigerant charge. a.; 15; c.; 35. b.; 20; d.; 50. |
| 64. |  |  |  |  |  |  |  |  |  |  | A major repair is defined by the EPA as replacing the: a.; condenser fan; c.; evaporator. b.; compressor; d.; B & C. |
| 65. |  |  |  |  |  |  |  |  |  | Exceptions to the required evacuation levels for recovery equipment that require an appliance be evacuated to only 0 PSIG apply to appliances that: a.; are being salvaged; c.; have defective fan motors. b.; are filled with water or substances that would damaged the recovery equipment; d.; have air cooled condensers. |
| 66. |  |  |  |  |  |  |  |  | The generally accepted rule for maximum liquid filling of a recovery cylinder is a.; 50%; c.; 80%. b.; 65%; d.; 95%. |
|  |  |  |  |  |  |  |  | 67.; The refrigerant you would expect to find in a split system air conditioner manufactured before 1995 would be: a.; R-12; c.; R-22. b.; R-11; d.; R-502.. |
| 68. |  |  |  |  |  |  | System dependant recovery equipment is limited to appliances containing less than ___ pounds of refrigerant. a.; 15; c.; 50. b.; 35; d.; 65. |
| 69. |  |  |  |  |  | The state and condition of the refrigerant entering the receiver is: a.; superheated high pressure vapor; c.; superheated high pressure liquid. b.; sub-cooled high pressure vapor; d.; sub-cooled high pressure liquid. |
| 70. |  |  |  |  | The state and condition of the refrigerant leaving the accumulator is: a.; superheated high pressure vapor; c.; superheated low pressure vapor. b.; sub-cooled high pressure vapor; d.; sub-cooled high pressure vapor. |
| 71. |  |  |  |  | Deep vacuums used for evacuation and dehydration of refrigeration systems are measured in: a.; tons; c.; PSIA. b.; PSIG; d.; microns. |
| 72. |  |  |  | Non-condensables in an air conditioning or refrigeration system will: a.; tend to stay near the top of the condenser; c.; be purged with a recovery machine. b.; increase high side pressure; d.; all of the above. |
| 73. |  |  | R-134a is a drop-in replacement for: a.; R-12; c.; R-11. b.; R-22; d.; It is not a drop-in replacement. |
| 74. |  | One of the easiest ways to identify the type of refrigerant in a system is to: a.; check the TXV; c.; check the safety cut-out. b.; check the name plate data; d.; smell the refrigerant. |
| 75. | At a room temperature ambient of 80°F, the pressure of a cylinder containing enough R-22 to have liquid in it will be approximately ______ PSIG: a.; 143; c.; 114. b.; 74; d.; 212. |