Mobile Radiography

Communication

· Park mobile unit outside patient’s room

· Establish rapport

· Get patient’s permission to do the exam

· Explain the procedure

· Move items that will be in the way

· Incognizant patients

· Surgery/Emergency Room/ICU

Demonstration of Air-Fluid Levels

Demonstration of Air-Fluid Levels

Types of Equipment

· Power supplies

· Generators

· Power drive

Power Supplies

· Portable light duty units

· 220V or 110V outlet

· Full power mobile institutional units

· Capacitor discharge

· Battery operated

Generators

· Capacitor discharge

· Constant potential output

· Battery operated 

· 3 phase output

Power Drive

· Self-propulsion for mobile unit

· Dead-man switch

· Must use caution when piloting equipment

Special Technical Factor Selection Considerations

· Kilovoltage

· Milliampere-seconds

· Distance

· Grids

· Film/screen combinations

· Other factors

Milliampere-Seconds

· Low power units not capable of high mAs techniques needed for grid radiography

· Double or triple exposure

· Be careful not to overload tube

Distance

· Must be measured accurately

· 40”

· 56”

· 72”

Grids

