COMBINATION VEHICLE (PRACTICE TEST)


NAME: ________________________ 

1  A_ What part of the kingpin should the locking jaws close around? 


A 
The shank

B
The head


C
The base

2____ Why should you lock the glad hands or dummy couplers to each other when you are not 
towing another vehicle?


A
The air cycles back, getting cleaner with each cycle.


B
The connected brake circuit becomes a backup air tank.


C
This will keep dirt and water out of the lines.

3____ If you are towing a trailer that blocks your view to the rear, the law requires 


A
That you have mirrors on both the left and right-hand outside


B
That you have mirrors on the left-hand side only


C
That you have mirrors on the right-hand side only

4____ Which one of the following statements is true?


A
The brake and suspension systems of combination vehicles are most 



effective with a light load.


B
Light vehicles need more braking power to stop than heavy vehicles


C
“Bobtail” tractors take longer to stop than loaded combination vehicles
5____ Using the trailer hand valve (Johnson/trolley valve) before the brake pedal to prevent 
trailer skids in normal driving:


A
Should not be done

B
Results in the least amount of skidding


C
Is the best method of straight-line backing

6____ There are two things that a driver can do to prevent a rollover. They are (1) go slow 
around corners and (2) 


A
Make sure that the brakes are properly balanced


B
Keep both hands firmly on the steering wheel


C
Keep the weight of your cargo as close to the ground as possible
7____ Trailer made before 1975, equipped with air brakes;


A
often do not have spring brakes

B
are heavier and safer to use


C
Usually need a glad hand converter

8____ When you get ready to back under the trailer, line up;


A
About 12 degrees off the line of the trailer


B
The kingpin to engage the driver’s side locking jaw first


C
Directly in front of the trailer
9____ You are coupling a tractor to a trailer and have backed up to it, but not under it. What   

           should you hook up before backing fully under the trailer


A
The safety chain


B
The emergency and service air lines


C
Nothing, back up and lock the fifth wheel

10___ You have coupled a tractor with a trailer. Where should the landing gear (Front trailer 
supports) be before driving away?


A
1/2 raised with the crank handle secured in the keeper


B
3/4 raised with the crank handle secured in the keeper


C
Fully raised with the crank handle secured in the keeper

11___ You are driving a semi and the trailer breaks away, pulling apart the air lines. You expect 

the trailer brakes to come on and;


A
The tractor to lose all air pressure


B
The tractor brakes to keep working

C
The trailer supply valve to stay open

12___ A device which corrects for brake lag on vehicle with air brakes is the 


A
Quick release valve


B
Relay valve

C
Front wheel limiting valve

13___ A full trailer is 10 times more likely to rollover than an empty one.   (True-False)

14___ A balanced load is easier to stop than one that is not balanced?    (True-False)

15___ What is a good rule for preventing rollovers?


A
Keep the load close to the ground


B
Take it slow


C
Steer gently


D
All of the above
16___ The longer the unit, the more rearward amplification you have?   (True-False)

17___ The fewer axles on a unit, the more stable it is?     (True-False)
18___If you have a trailer jackknife, you should use the trailer hand valve (Johnson/trolley 
valve to straighten your vehicle out?      (True-False) 

19___ Rearward amplification, causes small movements (turns) at the front of the vehicle to
become larger movements side to side at the back of the vehicle? 
(True-
False)

20___ When turning a corner with a set of doubles, which wheels will off-track the most?


A
Tractor drive wheels


B
Converter dolly wheels


C
Rear wheels on the second trailer

D
Rear wheels on the first trailer

21__?_ When making a right hand turn with doubles, keep the rear wheels of the first trailer 
close to the curb.   (True-False)
22___ When you turn suddenly while pulling triples, which trailer is more likely to rollover?


A
The first trailer


B
All rollover at the same time


C
The third trailer

D
The second trailer

23___ If you do not have spring brakes on your trailer and you cross the air lines;


A
Your trailer brakes will not release


B
If all of the air has leaked off, you will not have any trailer brakes

C 
It will not affect your brakes at all

24___ The relay valve connects the trailer air tanks to the trailer air brakes? (True-False)

25___ You should look into the back of the fifth wheel to see if the jaws are locked around the   

            shank of the kingpin  
(True-False)

26___ When completely hooked up to a set of doubles, how do you check your air lines?


A
Look at you air supply gauge


B
Chock your wheels, release brakes, open back shut off valves

C
Push both valves in and drive off, no there is no need to check the system

27___ What is a converter dolly?


A
Extra set of wheels for weight distribution


B
Portable fifth wheel

C
The piece that hooks the tractor to the first trailer

28___ Never unlock the pintle hook with the converter dolly still under the rear trailer? (True-
False)

29___With doubles you have a better chance of skidding or loss of traction in snow than just the 
tractor trailer     (True-False)

30___ A bobtail takes longer to stop than a fully loaded tractor-trailer combination. (True-
False)

31___If a trailer does not have spring brakes, the parking brakes will hold;


A
As long as there is air in the tanks

B
Whether there is air in the system or not


C
Not at all

32___ Why is it necessary to look far ahead while driving?


A
To prevent rollover


B
To be able to change lanes gradually


C
To prevent jackknifing


D 
All of the above
END

PAGE  
1
c: comb vehicle practice\rbs\7-5-05      1 of 3

