Name: ______________________

Electricity Review Sheet

True or False

- T   F   The black wire is also known as the cold or neutral wire

- T   F   The white wire is also known as the cold or neutral wire

- T   F   The bare or copper wire is the grounding wire.

- T   F   If a cord is cut you can just put electrical tape on the cut regardless of the severity.

- T   F   Every electrical wire should be treated as hot to prevent injury.

- T   F   The third prong on a three-way wire system is for the hot wire

- T   F   The wire should be put onto the screw in a counterclockwise fashion.

- T   F   Roughing in means to install wires before the dry wall is installed.

- T   F   The breaker should be turned off when fixing or repairing all electrical work.

- T   F   North is the negative side of a magnet.

Multiple Choice

- What does GFCI stand for?

  - Ground Fault Circuit Interrupter

  - Ground Failure Circuit Interrupter

  - Gravity Failure Current Interrupter

  - Ground Fault Current Instigator

- GFCI’s must be installed in all of the following areas except

  - Decks

  - Bathrooms

  - Living Rooms

  - Construction Sites

- Muscles will contract and not release at what amperage?

  - .2 amps

  - .02 amps

  - 2 amps

  - 20 amps

- What color screw should a hot wire be screwed to?

  - Brass

  - Silver

  - Green

  - Doesn’t matter

- What color screw should a neutral wire be screwed to?

  - Brass

  - Silver

  - Green

  - Doesn’t matter

- How much insulation should be stripped from the end of the wire?

  - ½ inch

  - 1 inch

  - 3 inches

  - ¾ inch

- The following is also knows as amps or amperage

  - Watt

  - Kilowatt

  - Current

  - Volts

- What is the movement of free electrons through a conductor?

  - Watt

  - Current

  - Amp

  - Electricity

- Which is the formula for watts?

  - Amps * Volts

  - Amps/volts

  - Kilowats * Volts

  - Volts/Amps

- How many watts are in a kilowatt?

  - 100

  - 10

  - 1

  - 1000

Short Answer

- What three major points determine how much physical damage you can receive from an electrical shock? (6pts)

- What are three good insulators of electricity? (6pts)

- What are two good conductors of electricity? (4pts)

- If you are using metal boxes what should you do with your grounding wires? (2pts)

- What is the symbol for ohms, voltage, and amperage? Please label your symbols. (6pts)

- A refrigerator motor rated at 4 amps that consumes 340 watts should be plugged into a circuit containing how many volts? (2 pts)

- In a 120 volt circuit, how many amps will 3 1000- watt heaters use? (2 pts)

- There are 4 microwaves running on 240 volts, using 40 amps how many KILOWATTS will they consume? (2 pts)