Threads and Thread Cutting

Threads

- ______________

- Helical ridge of uniform section formed on inside or outside of cylinder or cone

- Used for several purposes:

- Fasten devices such as ________________________________________

- Provide accurate measurement, as in micrometer

- Transmit motion

- _________________________

Thread Terminology

- ____________________

- Helical ridge of uniform section formed on inside or outside of cylinder or cone

- External thread

- Cut on external surface or cone

- ____________________

- Produced on inside of cylinder or cone

- Pitch diameter

- Diameter of imaginary cylinder that passes through thread at point where groove and thread widths are equal

- Equal to major diameter minus single depth of thread

- Tolerance and allowances given at pitch diameter line

- ________________

The distance from a given point on one thread to a corresponding point on the very next thread

- ___________-	The distance a screw thread advances in one revolution.

- The lead and the pitch of a single lead thread are the same.

- On double lead threads, the lead is twice the pitch.

- A double lead thread has two start points.

- _____________

- Bottom surface joining sides of two adjacent threads

- External thread on minor diameter

- Internal thread on major diameter

- _______________

- Top surface joining two sides of thread

- External thread on major diameter

- Internal thread on minor diameter

- ______________

- Thread surface that connects crest with root

- Depth of thread

- Distance between crest and root measured perpendicular to axis

- ________________

- Included angle between sides of thread measured in axial plane

- Helix angle

- Angle that thread makes with plane perpendicular to thread axis

- _____________________

- Helical ridge of uniform cross section onto which nut is threaded in clockwise direction

- When cut on lathe, toolbit advanced from right to left

- ___________________

  - Helical ridge of uniform cross section onto which nut is threaded in counterclockwise direction

  - When cut on lathe, toolbit advanced from left to right