Push Stick

Agriculture I

Hand Tools Project

Materials Needed:

- 1 piece unsurfaced lumber 1”x 3”x 12”

Procedure

- Surface the 1” board to 7/8” thickness with a hand plane

- Cut piece to length as shown on the bill of materials

- Lay off 1” squares on a piece of heavy paper and sketch an outline of the push stick, as shown in the drawing.

- Cut out the pattern to use as a template

- Lay out the push stick by marking around the template

- Cut out the push stick by sawing along the outside edges of the mark with a hand saw or coping saw

- Chamfer edges with a hand plane

- Sand and finish.

Rubric:

Possible		Earned

- Push stick is cut to scale			20			_____

- Board is planed to cored thickness	10			_____

- Template is laid out correctly		20			_____

- Edges have been sanded down		20			_____

- Edges have been chamfered		10			_____

- Push stick has been finished		20			_____

Total				100			_____